CONVOLUTION

Code:

import numpy as np

import matplotlib.pyplot as plt

# -----------------------------

# USER INPUT

# -----------------------------

x = list(map(int, input("Enter signal x(n) (space separated): ").split()))

origin_x = int(input(f"Enter index of origin (n=0) for x(n) [0 to {len(x)-1}]: "))

h = list(map(int, input("Enter signal h(n) (space separated): ").split()))

origin_h = int(input(f"Enter index of origin (n=0) for h(n) [0 to {len(h)-1}]: "))

x = np.array(x)

h = np.array(h)

# -----------------------------

# CONVOLUTION

# -----------------------------

y = np.convolve(x, h)

# Time index calculation

n_start = -(origin_x + origin_h)

n_end = n_start + len(y) - 1

n = np.arange(n_start, n_end + 1)

# Time index for x and h

nx = np.arange(-origin_x, len(x) - origin_x)

nh = np.arange(-origin_h, len(h) - origin_h)

# -----------------------------

# PLOTTING

# -----------------------------

plt.figure(figsize=(12, 8))

plt.subplot(3, 1, 1)

plt.stem(nx, x, basefmt="k")

plt.title("Signal x(n)")

plt.xlabel("n")

plt.ylabel("Amplitude")

plt.grid(True)

plt.subplot(3, 1, 2)

plt.stem(nh, h, basefmt="k")

plt.title("Signal h(n)")

plt.xlabel("n")

plt.ylabel("Amplitude")

plt.grid(True)

plt.subplot(3, 1, 3)

plt.stem(n, y, basefmt="k")

plt.title("Convolution y(n) = x(n) * h(n)")

plt.xlabel("n")

plt.ylabel("Amplitude")

plt.grid(True)

plt.tight_layout()

plt.show()

# -----------------------------

# PRINT RESULTS

# -----------------------------

print("\nConvolution Result:")

print("n :", n)

print("y(n):", y)

Output: