NYQUIST SAMPLING THEOREM
code:
import pygame

import sys

import math

# -----------------------------

# USER INPUT

# -----------------------------

fs = float(input("Enter sampling frequency fs (Hz): "))

# -----------------------------

# SIGNAL PARAMETERS

# -----------------------------

fmax = 4.0                     # sin(8πt)

Ts = 1 / fs

TIME_WINDOW = 1.0

WIDTH = 1000

HEIGHT = 1000

AMPLITUDE = 90

FPS = 60

# -----------------------------

# COLORS

# -----------------------------

WHITE = (255, 255, 255)

BLACK = (0, 0, 0)

RED   = (255, 0, 0)    # Original

BLUE  = (0, 0, 255)    # Sampled

GREEN = (0, 180, 0)    # Reconstructed

GRAY  = (150, 150, 150)

# -----------------------------

# SIGNAL FUNCTIONS

# -----------------------------

def original_signal(t):

return math.sin(8 * math.pi * t)

def sinc(x):

return 1 if x == 0 else math.sin(math.pi * x) / (math.pi * x)

def reconstruct(t, samples):

y = 0

for tn, xn in samples:

y += xn * sinc((t - tn) / Ts)

return y

# -----------------------------

# PYGAME INIT

# -----------------------------

pygame.init()

screen = pygame.display.set_mode((WIDTH, HEIGHT))

pygame.display.set_caption("Nyquist Sampling Theorem")

clock = pygame.time.Clock()

font = pygame.font.SysFont("Arial", 18)

center_top = HEIGHT // 4

center_bottom = 3 * HEIGHT // 4

# -----------------------------

# CREATE SAMPLES

# -----------------------------

samples = []

n = 0

while n * Ts <= TIME_WINDOW:

t = n * Ts

samples.append((t, original_signal(t)))

n += 1

# -----------------------------

# MAIN LOOP

# -----------------------------

while True:

clock.tick(FPS)

for event in pygame.event.get():

if event.type == pygame.QUIT:

pygame.quit()

sys.exit()

screen.fill(WHITE)

# -----------------------------

# Original signal (TOP)

# -----------------------------

orig_points = []

for x in range(WIDTH):

t = (x / WIDTH) * TIME_WINDOW

y = original_signal(t)

orig_points.append((x, center_top - int(y * AMPLITUDE)))

pygame.draw.lines(screen, RED, False, orig_points, 2)

pygame.draw.line(screen, GRAY, (0, center_top), (WIDTH, center_top), 1)

# -----------------------------

# Sampled signal (BOTTOM - stems)

# -----------------------------

pygame.draw.line(screen, GRAY, (0, center_bottom), (WIDTH, center_bottom), 1)

for t, y in samples:

sx = int((t / TIME_WINDOW) * WIDTH)

sy = center_bottom - int(y * AMPLITUDE)

pygame.draw.line(screen, BLUE, (sx, center_bottom), (sx, sy), 2)

pygame.draw.circle(screen, BLUE, (sx, sy), 4)

# -----------------------------

# Reconstructed signal (TOP)

# -----------------------------

recon_points = []

for x in range(WIDTH):

t = (x / WIDTH) * TIME_WINDOW

y = reconstruct(t, samples)

recon_points.append((x, center_top - int(y * AMPLITUDE)))

pygame.draw.lines(screen, GREEN, False, recon_points, 2)

# -----------------------------

# TEXT INFO

# -----------------------------

info = [

"Original Signal: sin(8πt)",

f"fmax = {fmax} Hz",

f"fs = {fs} Hz",

f"fs / fmax = {fs / fmax:.2f}",

"Reconstruction OK" if fs >= 2*fmax else "Aliasing Occurs"

]

for i, text in enumerate(info):

screen.blit(font.render(text, True, BLACK), (10, 10 + 20*i))

pygame.display.flip()

Output: