NYQUIST SAMPLING THEOREM
code:
import pygame

import sys

import math

# -----------------------------

# USER INPUT

# -----------------------------

fs = float(input("Enter sampling frequency fs (Hz): "))

# -----------------------------

# SIGNAL PARAMETERS

# -----------------------------

fmax = 4.0                     # sin(8πt)

Ts = 1 / fs

TIME_WINDOW = 1.0

WIDTH = 1000

HEIGHT = 1000

AMPLITUDE = 90

FPS = 60

# -----------------------------

# COLORS

# -----------------------------

WHITE = (255, 255, 255)

BLACK = (0, 0, 0)

RED   = (255, 0, 0)    # Original

BLUE  = (0, 0, 255)    # Sampled

GREEN = (0, 180, 0)    # Reconstructed

GRAY  = (150, 150, 150)

# -----------------------------

# SIGNAL FUNCTIONS

# -----------------------------

def original_signal(t):

return math.sin(8 * math.pi * t)

def sinc(x):

return 1 if x == 0 else math.sin(math.pi * x) / (math.pi * x)

def reconstruct(t, samples):

y = 0

for tn, xn in samples:

y += xn * sinc((t - tn) / Ts)

return y

# -----------------------------

# PYGAME INIT

# -----------------------------

pygame.init()

screen = pygame.display.set_mode((WIDTH, HEIGHT))

pygame.display.set_caption("Nyquist Sampling Theorem")

clock = pygame.time.Clock()

font = pygame.font.SysFont("Arial", 18)

center_top = HEIGHT // 4

center_bottom = 3 * HEIGHT // 4

# -----------------------------

# CREATE SAMPLES

# -----------------------------

samples = []

n = 0

while n * Ts <= TIME_WINDOW:

t = n * Ts

samples.append((t, original_signal(t)))

n += 1

# -----------------------------

# MAIN LOOP

# -----------------------------

while True:

clock.tick(FPS)

for event in pygame.event.get():

if event.type == pygame.QUIT:

pygame.quit()

sys.exit()

screen.fill(WHITE)

# -----------------------------

# Original signal (TOP)

# -----------------------------

orig_points = []

for x in range(WIDTH):

t = (x / WIDTH) * TIME_WINDOW

y = original_signal(t)

orig_points.append((x, center_top - int(y * AMPLITUDE)))

pygame.draw.lines(screen, RED, False, orig_points, 2)

pygame.draw.line(screen, GRAY, (0, center_top), (WIDTH, center_top), 1)

# -----------------------------

# Sampled signal (BOTTOM - stems)

# -----------------------------

pygame.draw.line(screen, GRAY, (0, center_bottom), (WIDTH, center_bottom), 1)

for t, y in samples:

sx = int((t / TIME_WINDOW) * WIDTH)

sy = center_bottom - int(y * AMPLITUDE)

pygame.draw.line(screen, BLUE, (sx, center_bottom), (sx, sy), 2)

pygame.draw.circle(screen, BLUE, (sx, sy), 4)

# -----------------------------

# Reconstructed signal (TOP)

# -----------------------------

recon_points = []

for x in range(WIDTH):

t = (x / WIDTH) * TIME_WINDOW

y = reconstruct(t, samples)

recon_points.append((x, center_top - int(y * AMPLITUDE)))

pygame.draw.lines(screen, GREEN, False, recon_points, 2)

# -----------------------------

# TEXT INFO

# -----------------------------

info = [

"Original Signal: sin(8πt)",

f"fmax = {fmax} Hz",

f"fs = {fs} Hz",

f"fs / fmax = {fs / fmax:.2f}",

"Reconstruction OK" if fs >= 2*fmax else "Aliasing Occurs"

]

for i, text in enumerate(info):

screen.blit(font.render(text, True, BLACK), (10, 10 + 20*i))

pygame.display.flip()

Output:

Operations on samples 
Code:
import pygame

import sys

# -----------------------------

# PYGAME SETUP

# -----------------------------

WIDTH = 1000

HEIGHT = 1000

WHITE = (255, 255, 255)

BLACK = (0, 0, 0)

RED   = (255, 0, 0)

BLUE  = (0, 0, 255)

pygame.init()

screen = pygame.display.set_mode((WIDTH, HEIGHT))

pygame.display.set_caption("Discrete-Time Signal Operations")

font = pygame.font.SysFont("Arial", 16)

clock = pygame.time.Clock()

# -----------------------------

# DRAW STEM PLOT

# -----------------------------

def draw_signal(n, x, center_y, color, label):

pygame.draw.line(screen, BLACK, (50, center_y), (WIDTH-50, center_y), 2)

pygame.draw.line(screen, BLACK, (WIDTH//2, 50), (WIDTH//2, HEIGHT-50), 2)

screen.blit(font.render(label, True, BLACK), (50, center_y - 150))

scale_x = 60

origin_x = WIDTH // 2

origin_y = center_y

if not x:

return

max_val = max(abs(val) for val in x)

for ni, xi in zip(n, x):

px = origin_x + ni * scale_x

py = origin_y - xi * 20

pygame.draw.line(screen, color, (px, origin_y), (px, py), 2)

pygame.draw.circle(screen, color, (px, py), 4)

screen.blit(font.render(str(xi), True, BLACK), (px - 10, py - 20))

screen.blit(font.render(str(ni), True, BLACK), (px - 10, origin_y + 5))

for y_val in range(-max_val-1, max_val+2):

py = origin_y - y_val * 20

if 0 < py < HEIGHT:

screen.blit(font.render(str(y_val), True, BLACK), (origin_x - 40, py - 5))

# -----------------------------

# USER INPUT

# -----------------------------

x = list(map(int, input("Enter discrete signal x(n) (space separated): ").split()))

origin_index = int(input(f"Enter index of origin (n=0) in range 0..{len(x)-1}: "))

n = [i - origin_index for i in range(len(x))]

print("\nMENU")

print("1. Time Shifting")

print("2. Time Reversal")

print("3. Time Reversal + Shifting")

print("4. Time Scaling")

print("5. Scalar Multiplication")

print("6. Addition of Two Signals")

print("7. Subtraction of Two Signals")

print("8. Multiplication of Two Signals")

choice = int(input("Enter your choice: "))

# -----------------------------

# OPERATIONS

# -----------------------------

if choice == 1:

k = int(input("Enter shift value k: "))

y = x[:]

n2 = [i + k for i in n]

title = f"Time Shifted (k = {k})"

elif choice == 2:

y = x[::-1]

n2 = [-i for i in n[::-1]]

title = "Time Reversed"

elif choice == 3:

k = int(input("Enter shift value k: "))

y = x[::-1]

n2 = [-(i) + k for i in n[::-1]]

title = f"Time Reversed + Shifted (k = {k})"

elif choice == 4:

a = int(input("Enter scaling factor a: "))

y = x[:]

n2 = [i * a for i in n]

title = f"Time Scaled (a = {a})"

elif choice == 5:

a = int(input("Enter scalar value: "))

y = [a * i for i in x]

n2 = n[:]

title = f"Scalar Multiplied (A = {a})"

elif choice in [6, 7, 8]:

print("\nEnter second signal x2(n)")

x2 = list(map(int, input("Enter discrete signal x2(n): ").split()))

origin_index2 = int(input(f"Enter index of origin for x2(n) 0..{len(x2)-1}: "))

n_second = [i - origin_index2 for i in range(len(x2))]

# Create common time axis WITHOUT modifying original x

n_min = min(min(n), min(n_second))

n_max = max(max(n), max(n_second))

n2 = list(range(n_min, n_max + 1))

# Create dictionaries for fast lookup

dict1 = dict(zip(n, x))

dict2 = dict(zip(n_second, x2))

y = []

for ni in n2:

val1 = dict1.get(ni, 0)

val2 = dict2.get(ni, 0)

if choice == 6:

y.append(val1 + val2)

title = "Addition: x(n) + x2(n)"

elif choice == 7:

y.append(val1 - val2)

title = "Subtraction: x(n) - x2(n)"

elif choice == 8:

y.append(val1 * val2)

title = "Multiplication: x(n) * x2(n)"

else:

print("Invalid choice")

pygame.quit()

sys.exit()

# -----------------------------

# PRINT RESULTS

# -----------------------------

print("\nOriginal Signal:")

print("n:", n)

print("x(n):", x)

print("\nProcessed Signal:")

print("n:", n2)

print("y(n):", y)

# -----------------------------

# DISPLAY LOOP

# -----------------------------

while True:

clock.tick(60)

for event in pygame.event.get():

if event.type == pygame.QUIT:

pygame.quit()

sys.exit()

screen.fill(WHITE)

draw_signal(n, x, HEIGHT//3, RED, "Original Signal x(n)")

draw_signal(n2, y, 2*HEIGHT//3, BLUE, title)

pygame.display.flip()

import pygame

import sys

# -----------------------------

# PYGAME SETUP

# -----------------------------

WIDTH = 1000

HEIGHT = 1000

WHITE = (255, 255, 255)

BLACK = (0, 0, 0)

RED   = (255, 0, 0)

BLUE  = (0, 0, 255)

pygame.init()

screen = pygame.display.set_mode((WIDTH, HEIGHT))

pygame.display.set_caption("Discrete-Time Signal Operations")

font = pygame.font.SysFont("Arial", 16)

clock = pygame.time.Clock()

# -----------------------------

# DRAW STEM PLOT

# -----------------------------

def draw_signal(n, x, center_y, color, label):

pygame.draw.line(screen, BLACK, (50, center_y), (WIDTH-50, center_y), 2)

pygame.draw.line(screen, BLACK, (WIDTH//2, 50), (WIDTH//2, HEIGHT-50), 2)

screen.blit(font.render(label, True, BLACK), (50, center_y - 150))

scale_x = 60

origin_x = WIDTH // 2

origin_y = center_y

if not x:

return

max_val = max(abs(val) for val in x)

for ni, xi in zip(n, x):

px = origin_x + ni * scale_x

py = origin_y - xi * 20

pygame.draw.line(screen, color, (px, origin_y), (px, py), 2)

pygame.draw.circle(screen, color, (px, py), 4)

screen.blit(font.render(str(xi), True, BLACK), (px - 10, py - 20))

screen.blit(font.render(str(ni), True, BLACK), (px - 10, origin_y + 5))

for y_val in range(-max_val-1, max_val+2):

py = origin_y - y_val * 20

if 0 < py < HEIGHT:

screen.blit(font.render(str(y_val), True, BLACK), (origin_x - 40, py - 5))

# -----------------------------

# USER INPUT

# -----------------------------

x = list(map(int, input("Enter discrete signal x(n) (space separated): ").split()))

origin_index = int(input(f"Enter index of origin (n=0) in range 0..{len(x)-1}: "))

n = [i - origin_index for i in range(len(x))]

print("\nMENU")

print("1. Time Shifting")

print("2. Time Reversal")

print("3. Time Reversal + Shifting")

print("4. Time Scaling")

print("5. Scalar Multiplication")

print("6. Addition of Two Signals")

print("7. Subtraction of Two Signals")

print("8. Multiplication of Two Signals")

choice = int(input("Enter your choice: "))

# -----------------------------

# OPERATIONS

# -----------------------------

if choice == 1:

k = int(input("Enter shift value k: "))

y = x[:]

n2 = [i + k for i in n]

title = f"Time Shifted (k = {k})"

elif choice == 2:

y = x[::-1]

n2 = [-i for i in n[::-1]]

title = "Time Reversed"

elif choice == 3:

k = int(input("Enter shift value k: "))

y = x[::-1]

n2 = [-(i) + k for i in n[::-1]]

title = f"Time Reversed + Shifted (k = {k})"

elif choice == 4:

a = int(input("Enter scaling factor a: "))

y = x[:]

n2 = [i * a for i in n]

title = f"Time Scaled (a = {a})"

elif choice == 5:

a = int(input("Enter scalar value: "))

y = [a * i for i in x]

n2 = n[:]

title = f"Scalar Multiplied (A = {a})"

elif choice in [6, 7, 8]:

print("\nEnter second signal x2(n)")

x2 = list(map(int, input("Enter discrete signal x2(n): ").split()))

origin_index2 = int(input(f"Enter index of origin for x2(n) 0..{len(x2)-1}: "))

n_second = [i - origin_index2 for i in range(len(x2))]

# Create common time axis WITHOUT modifying original x

n_min = min(min(n), min(n_second))

n_max = max(max(n), max(n_second))

n2 = list(range(n_min, n_max + 1))

# Create dictionaries for fast lookup

dict1 = dict(zip(n, x))

dict2 = dict(zip(n_second, x2))

y = []

for ni in n2:

val1 = dict1.get(ni, 0)

val2 = dict2.get(ni, 0)

if choice == 6:

y.append(val1 + val2)

title = "Addition: x(n) + x2(n)"

elif choice == 7:

y.append(val1 - val2)

title = "Subtraction: x(n) - x2(n)"

elif choice == 8:

y.append(val1 * val2)

title = "Multiplication: x(n) * x2(n)"

else:

print("Invalid choice")

pygame.quit()

sys.exit()

# -----------------------------

# PRINT RESULTS

# -----------------------------

print("\nOriginal Signal:")

print("n:", n)

print("x(n):", x)

print("\nProcessed Signal:")

print("n:", n2)

print("y(n):", y)

# -----------------------------

# DISPLAY LOOP

# -----------------------------

while True:

clock.tick(60)

for event in pygame.event.get():

if event.type == pygame.QUIT:

pygame.quit()

sys.exit()

screen.fill(WHITE)

draw_signal(n, x, HEIGHT//3, RED, "Original Signal x(n)")

draw_signal(n2, y, 2*HEIGHT//3, BLUE, title)

pygame.display.flip()

Output: